# Baja SAE IMU Data Logger — Full Build & Replication Guide

Project: Raspberry Pi 4B + WitMotion WT901 IMU logging system
Purpose: In-vehicle accelerometer / gyroscope / angle / magnetometer data capture with GPIO-controlled start/stop buttons and LED indicators, auto-starting on boot via systemd.
Status as documented: Fully operational. First deployed March 2026.
Target audience: Baja team members replicating or extending this system from scratch.

No internet required. This system operates completely offline once built. There is no cloud dependency, no API key, no WiFi needed in the field. The Pi logs to its SD card. To retrieve data after a run, either SSH in over a local network and scp the files off, or pull the SD card and mount it directly on any computer using AnyLinuxFS (macOS/Windows) or native Linux filesystem tools. Log files live at /home/m1000/wt901/witmotion_raw.log and parsed CSVs in the same directory. If you parse a raw log, it will return the parsed csv with timestamps.

## Table of Contents

- Hardware List & Wiring

- Raspberry Pi OS Setup

- Serial Port Configuration

- Python Virtual Environment & Dependencies

- Project File Structure

- Software Architecture Overview

- The Bootloader (bootloadervw.py)

- The IMU Logger (imu_logger2.1.py)

- The Shell Wrapper (run_bootloader.sh)

- systemd Services

- Data Parsing (witmotion_log_parser-2.py)

- Sensor Mount & Axis Correction

- Troubleshooting Log — Problems Solved

- Calibration Procedure

- Testing the System

- Future Improvements Roadmap

- Quick Reference Cheat Sheet

## 1. Hardware List & Wiring

### Components

| Item | Notes |
| --- | --- |
| Raspberry Pi 4B (8 GB RAM) | Any 4B variant works; 8 GB is comfortable headroom |
| WitMotion WT901 IMU | 9-axis: accelerometer, gyroscope, magnetometer + onboard angle fusion |
| GPIO terminal/header breakout board | Simplifies reliable wiring vs. bare header pins |
|  | Boot indicator and armed/logging indicator |
| Momentary push buttons (2–3x) | START, STOP, and optionally a dedicated safe-shutdown button |
| Serial wiring | 3-wire: TX, RX, GND between Pi and WT901 |
| Micro-SD card (16 GB+) | Class 10 or better |
| 5V power supply (3A minimum) | Underpowering causes random crashes — do not cheap out here |

### GPIO Pin Assignments (BCM numbering)

| Signal | BCM Pin | Notes |
| --- | --- | --- |
| BOOT_LED | GPIO 26 | Solid on = bootloader is alive and running |
| ARMED_LED | GPIO 19 | Blinks at 1 Hz = ready and waiting for START button |
| LOG_LED | GPIO 21 | Blinks at 4 Hz = actively logging IMU data |
| SHUTDOWN_LED | GPIO 18 | Blinks at 10 Hz during graceful shutdown sequence |
| START button | GPIO 13 | Active LOW (internal pull-up enabled in software) |
| STOP button | GPIO 17 | Active LOW (internal pull-up enabled in software) |
| SHUTDOWN button | GPIO 16 | Active LOW — triggers graceful OS shutdown |
| WT901 TX → Pi RX | GPIO 15 (RXD) | Yellow wire from WT901 TX |
| WT901 RX → Pi TX | GPIO 14 (TXD) | Green wire to WT901 RX |
| GND | Any GND terminal | Purple wire — shared ground to WT901 |
| 5V power | 5V pin | Red wire — VCC to WT901 |

### LED Behavior Summary

Understanding the LED states at a glance is critical in the field:

| LED | GPIO | Pattern | Meaning |
| --- | --- | --- | --- |
| BOOT | 26 | Solid ON | System is booted and bootloader is running |
| ARMED | 19 | 1 Hz blink | Waiting for you to press START |
| LOG | 21 | 4 Hz blink | Actively recording IMU data |
| SHUTDOWN | 18 | 10 Hz blink | Shutdown sequence in progress — wait for it to finish before cutting power |

Always wait for all LEDs to go dark before removing power. The 10 Hz shutdown blink on GPIO 18 is your signal that the OS is in the process of halting. Cutting power during this window risks SD card corruption.

### Wiring Notes

The enclosure uses a 3D-printed case with a GPIO terminal/screw-block breakout board mounted on top of the Pi. This board exposes every GPIO as a labeled screw terminal, which makes field wiring and re-wiring far more reliable than bare header pins. Strongly recommended for any vehicle-mounted build.

- WT901 power: The WT901 is powered from the Pi’s 5V pin via the red wire. This is fine for bench use. In the car, consider whether the Pi’s 5V rail is stable enough under vibration and load — if you see IMU dropouts, power the WT901 from a separate regulated supply.

- WT901 logic levels: The WT901 communicates at 3.3V logic over serial. The Pi’s UART TX/RX are also 3.3V. No level shifter is needed. Do not connect the WT901 RX pin to a 5V signal — it will damage the sensor.

- Buttons: Wire each button between its GPIO pin and any GND terminal. The Pi’s internal pull-up resistors are enabled in software — no external resistors are needed for the buttons.

## 2. Raspberry Pi OS Setup

### Flash the OS

Use the Raspberry Pi Imager tool (https://rpi.imager.org) to flash Raspberry Pi OS FULL (64-bit) to your SD card.

In the Imager’s advanced settings (gear icon) before flashing:

- Set hostname (e.g., baja-logger) (M1000 PREFERRED FOR PATHS)

- Enable SSH

- Set your username (this guide uses m1000 — adjust all paths if you use a different username)

- Set WiFi credentials (useful for first-time setup; can be removed later)

- Set locale/timezone

### First Boot & Update

sudo apt update && sudo apt upgrade -y
sudo apt install -y python3-pip python3-venv git screen

### Create the project user (if not already m1000)

All paths in the scripts are hardcoded to /home/m1000/wt901/. If you use a different username, you must update the paths in every script and service file. It is easiest to just create this user or adjust everything up front.

# If your Pi user is not m1000, create a symlink or find-replace all paths:
sudo useradd -m -s /bin/bash m1000
sudo usermod -aG gpio,dialout m1000

If you keep your own username, add yourself to the required groups:

sudo usermod -aG gpio,dialout $USER
# Log out and back in for group changes to take effect

## 3. Serial Port Configuration

This is one of the most critical and error-prone steps. The Pi has two UARTs: a full hardware UART (PL011) and a mini-UART. By default, the mini-UART is assigned to ttyS0 and used for the serial console. The WT901 requires the full hardware UART for reliable communication.

### Step 1 — Disable the serial console

sudo raspi-config

Navigate to: Interface Options → Serial Port

- “Would you like a login shell accessible over serial?” → No

- “Would you like the serial port hardware to be enabled?” → Yes

This disables the console on the serial pins but keeps the hardware port active.

### Step 2 — Disable Bluetooth to free the full UART

The Pi 4’s PL011 (full hardware UART) is assigned to Bluetooth by default. To reclaim it for serial use, add the following to /boot/config.txt (or /boot/firmware/config.txt on newer OS versions):

dtoverlay=disable-bt

Then disable the Bluetooth service:

sudo systemctl disable hciuart
sudo systemctl disable bluetooth

Reboot: sudo reboot

### Step 3 — Identify your serial port

After reboot:

ls -la /dev/serial* /dev/ttyAMA* /dev/ttyS*

You should see /dev/ttyAMA0 pointing to the hardware UART. The working port in this build is:

- Primary: /dev/ttyAMA0 (used in imu_logger2.1.py)

- Alternate: /dev/ttyS0 (used in imu_logger4_API.py — verify which port your system assigns)

Test that the port exists and you have access:

ls -la /dev/ttyAMA0
# Should show: crw-rw---- 1 root dialout ...
# Your user must be in the 'dialout' group

### Step 4 — Verify baud rate

The WT901’s default baud rate from the factory is 9600 baud. This is what the system uses. Do not change it in the WT901 firmware unless you explicitly need higher throughput, because changing it incorrectly will make the device appear dead (see “Bricking the Device” in the Troubleshooting section).

## 4. Python Virtual Environment & Dependencies

A virtual environment (venv) is essential here. System Python on Raspberry Pi OS is managed by the OS package manager, and installing packages globally with pip will fail or conflict on modern Debian-based systems.

### Create the venv

mkdir -p /home/m1000/wt901
cd /home/m1000/wt901
python3 -m venv .
# This creates bin/, lib/, include/ inside /home/m1000/wt901/

The venv Python binary is now at /home/m1000/wt901/bin/python. This exact path is hardcoded in the bootloader and shell scripts — do not change the venv location.

### Install dependencies

/home/m1000/wt901/bin/pip install witmotion RPi.GPIO

The witmotion package is the community Python library for WitMotion sensors. It wraps the serial protocol and provides message callbacks. The RPi.GPIO package controls GPIO pins.

Verify the install:

/home/m1000/wt901/bin/python -c "import witmotion; import RPi.GPIO; print('OK')"

### Install the witmotion-debug CLI tool

The imu_logger2.1.py script uses a command-line binary called witmotion-debug. This is installed as part of the witmotion Python package and should appear at:

/home/m1000/wt901/bin/witmotion-debug

Verify it exists:

ls -la /home/m1000/wt901/bin/witmotion-debug

Test it manually (Ctrl+C to stop):

/home/m1000/wt901/bin/witmotion-debug --path /dev/ttyAMA0 --verbose

You should see a stream of lines like:

DEBUG:witmotion:state: idle -> header, got 0x55
DEBUG:witmotion:state: header -> payload, got code 0x51
INFO:witmotion.cmd.debug:acceleration message - vec:(0.129, -7.675, -8.053) temp_celsius:35.28

If you see nothing or garbage characters, your serial port or baud rate is wrong (see Troubleshooting).

## 5. Project File Structure

/home/m1000/wt901/
├── bin/                          # venv binaries (python, pip, witmotion-debug, etc.)
├── lib/                          # venv libraries
├── include/                      # venv headers
├── bootloadervw.py               # GPIO supervisor — waits for button, launches logger
├── imu_logger2.1.py              # Core logger — reads IMU, writes CSV and raw log
├── imu_logger4_API.py            # Alternative logger using witmotion Python API directly
├── run_bootloader.sh             # systemd wrapper script — sets PATH, launches bootloader
├── witmotion_log_parser-2.py     # Post-run parser — converts raw log to clean CSV
├── witmotion_raw.log             # Raw log output (appended each session)
└── witmotion_parsed.csv          # Parsed CSV output (appended each session)

/etc/systemd/system/
├── wt901_bootloader.service      # Auto-starts the bootloader on boot
└── shutdown-monitor.service      # Monitors a GPIO button for safe shutdown

## 6. Software Architecture Overview

The system has a layered design with clear separation of concerns:

systemd
  └── wt901_bootloader.service
        └── run_bootloader.sh          (sets environment, launches with venv Python)
              └── bootloadervw.py      (GPIO supervisor)
                    └── imu_logger2.1.py  (spawned as subprocess when START is pressed)
                          └── witmotion-debug CLI  (spawned as subprocess, streams IMU data)

Why this layered approach?

- systemd manages auto-start, restart on failure, and logging to journald.

- run_bootloader.sh solves the PATH problem: systemd runs in a minimal environment and cannot find the venv Python without explicit path setting.

- bootloadervw.py runs continuously, managing the LED states and waiting for user input. It does GPIO cleanup and re-init between sessions so the logger starts fresh.

- imu_logger2.1.py is a child process. When the STOP button is pressed, the logger exits cleanly, and the bootloader returns to its waiting state — ready for the next run without requiring a reboot.

- The logger itself spawns witmotion-debug as a subprocess and reads its stdout line-by-line, parsing and writing to CSV.

Data flow:

WT901 IMU (serial) → witmotion-debug CLI → stdout pipe
                                                ↓
                                     imu_logger2.1.py
                                       ├── witmotion_raw.log    (every line, raw)
                                       └── witmotion_parsed.csv (parsed frames)
                                                ↓ (post-run)
                                   witmotion_log_parser-2.py
                                       └── <name>_parsed_<timestamp>.csv (clean, corrected)

## 7. The Bootloader (bootloadervw.py)

The bootloader is the persistent supervisor process. It starts on boot and never exits unless the Pi is shut down.

Behavior:

- Turns on BOOT_LED (solid) to indicate the system is alive.

- Blinks ARMED_LED at ~1 Hz, waiting for the START button.

- On START press: turns off ARMED_LED, calls GPIO.cleanup() to release all pins, then spawns imu_logger2.1.py as a subprocess and blocks until the logger exits.

- After the logger exits: re-initializes GPIO and returns to step 2.

Why GPIO.cleanup() before spawning the logger?

The logger also uses GPIO. If the bootloader holds the GPIO resources when the logger starts, you get permission errors or undefined pin states. Cleanup releases the pins so the logger can claim them cleanly. After the logger exits, the bootloader re-initializes its own pins.

Key code excerpt:

PYTHON_BIN = "/home/m1000/wt901/bin/python"
LOGGER_PATH = "/home/m1000/wt901/imu_logger2.1.py"

GPIO.cleanup()  # Release pins before spawning child
logger_process = subprocess.Popen([PYTHON_BIN, LOGGER_PATH])
logger_process.wait()  # BLOCK until logger exits

Why hardcode the venv Python path?

When spawned by systemd, the PATH environment variable is minimal. Using /usr/bin/python3 would use the system Python, which doesn’t have witmotion or RPi.GPIO installed in the venv. Hardcoding the venv binary ensures the correct interpreter and packages are always used.

## 8. The IMU Logger (imu_logger2.1.py)

The logger is the core data capture process. It is designed to be short-lived — one run per button press/release cycle.

Startup sequence:

- Sets up GPIO pins (START, STOP, LOG_LED).

- Opens raw log and CSV files in append mode.

- Waits for the START button press.

- Spawns witmotion-debug as a subprocess, capturing stdout.

- Enters the main loop.

Main loop:

- Toggles LOG_LED at ~4 Hz (fast blink = actively logging).

- Checks STOP button — exits cleanly if pressed.

- Uses select.select() for non-blocking reads from the subprocess stdout. This avoids blocking the button check and LED blink.

- Writes every raw line to witmotion_raw.log.

- Parses lines containing "payload" for packet type and data, writes to CSV.

Log file behavior — append mode:

Both files are opened with "a" (append). This means each session adds to the existing file. The raw log grows indefinitely; the parser can handle multi-session logs. If you want per-session files, change the open mode to "w" or add a timestamp to the filename.

CSV columns written in real-time:

timestamp, type, data, checksum_status

Note: this is a simplified on-the-fly CSV. The full clean CSV with axis-corrected data, proper column names, and merged frames is produced by the post-run parser.

Serial port used: /dev/ttyAMA0 at 9600 baud (the default from witmotion-debug).

## 9. The Shell Wrapper (run_bootloader.sh)

This script exists purely to solve the systemd PATH problem.

VENV_PYTHON="/home/m1000/wt901/bin/python"
BOOTLOADER_PY="/home/m1000/wt901/bootloadervw.py"

export PATH="/home/m1000/wt901/bin:$PATH"
echo "$(date) - Starting bootloader via run_bootloader.sh" >> "$RAW_LOG"

exec "$VENV_PYTHON" "$BOOTLOADER_PY" >> "$RAW_LOG" 2>&1

Key points:

- export PATH=... prepends the venv bin directory so any subprocess that looks up executables by name (like witmotion-debug) finds the venv version first.

- exec replaces the shell process with Python, so systemd tracks the Python PID directly.

- Stdout and stderr are redirected to the raw log, so bootloader startup messages are recorded.

- The echo "$(date) - Starting..." line creates visible session boundaries in the log file, making it easy to find where each boot begins.

## 10. systemd Services

### wt901_bootloader.service

Place this file at /etc/systemd/system/wt901_bootloader.service:

[Unit]
Description=WT901 IMU Bootloader Supervisor
After=network.target

[Service]
Type=simple
User=m1000
Group=m1000
WorkingDirectory=/home/m1000/wt901
ExecStart=/home/m1000/wt901/run_bootloader.sh
Restart=on-failure
Environment="PATH=/home/m1000/wt901/bin:/usr/bin:/bin"
StandardOutput=journal
StandardError=journal

[Install]
WantedBy=multi-user.target

Important notes:

- After=network.target is listed but not strictly required for IMU logging. It ensures any network-dependent services are up first — useful if you later add telemetry.

- Restart=on-failure restarts the bootloader only if it crashes. It does not restart it on a clean exit (which is correct — you don’t want it restarting during a clean shutdown).

- The duplicate Restart= line in the original file (Restart=always followed by Restart=on-failure) — only the last one takes effect. In the current file, that is on-failure. Clean this up to avoid confusion.

- Environment="PATH=..." is a belt-and-suspenders measure alongside what run_bootloader.sh does. Both are needed.

### shutdown-monitor.service

Place at /etc/systemd/system/shutdown-monitor.service:

[Unit]
Description=Shutdown Button Monitor
After=multi-user.target

[Service]
Type=simple
ExecStart=/usr/bin/python3 /home/pi/imu-datalogger/shutdown_monitor.py
WorkingDirectory=/home/pi/imu-datalogger
Restart=on-failure
RestartSec=5s
User=pi
Group=pi
StandardOutput=journal
StandardError=journal

[Install]
WantedBy=multi-user.target

Note: This service still references /home/pi/imu-datalogger/ and User=pi. This is from an earlier version of the project before the username was changed to m1000. Before using this service, update the paths and username to match your system.

The shutdown_monitor.py script monitors GPIO 16 for a button press (active LOW). When triggered, it flashes the GPIO 18 LED at 10 Hz as a visual warning that shutdown is in progress, then calls sudo shutdown -h now. Always wait for all LEDs to go dark before cutting vehicle power — removing power during the shutdown flash risks SD card corruption and a broken filesystem.

### Enabling and managing the services

# Enable and start
sudo systemctl daemon-reload
sudo systemctl enable wt901_bootloader.service
sudo systemctl start wt901_bootloader.service
sudo systemctl enable shutdown-monitor.service
sudo systemctl start shutdown-monitor.service

# Check status
sudo systemctl status wt901_bootloader.service

# View live logs
sudo journalctl -u wt901_bootloader.service -f

# Stop for debugging
sudo systemctl stop wt901_bootloader.service

# Restart after changes
sudo systemctl restart wt901_bootloader.service

## 11. Data Parsing (witmotion_log_parser-2.py)

After a logging session, the raw log contains a stream of DEBUG and INFO lines from the witmotion-debug tool. The parser converts this into a clean, analysis-ready CSV.

### Running the parser

# Parse the default raw log
/home/m1000/wt901/bin/python witmotion_log_parser-2.py

# Specify input and output files
/home/m1000/wt901/bin/python witmotion_log_parser-2.py witmotion_raw.log my_run_parsed.csv

### What the parser does

- Reads both new-format lines (with [HH:MM:SS.mmm] timestamp prefix) and legacy lines (no timestamp).

- Groups acceleration, gyro, angle, and magnetometer messages into complete frames anchored on each acceleration message.

- Applies mount-orientation corrections (see Section 12).

- Writes a CSV with both raw and corrected columns.

- Prints a summary to the terminal: frame count, duration, Hz rate, min/max/avg for all axes.

### Output CSV columns

| Column | Description |
| --- | --- |
| frame | Frame index |
| timestamp_iso | Full ISO timestamp (if available) |
| time_hms | HH:MM:SS.mmm |
| raw_ax_g .. raw_az_g | Raw accelerometer XYZ in g |
| raw_gx_dps .. raw_gz_dps | Raw gyroscope XYZ in °/s |
| raw_roll_deg .. raw_yaw_deg | Raw euler angles in ° |
| accel_lat_g .. accel_vert_g | Mount-corrected acceleration |
| gyro_x_dps .. gyro_z_dps | Mount-corrected gyro |
| pitch_deg, roll_deg, yaw_deg | Mount-corrected angles |
| mag_x, mag_y, mag_z | Magnetometer raw counts |
| temp_c | IMU die temperature |

### Legacy log warning

If the log was captured before timestamp prefixes were added to the logger, the parser will warn:

⚠  Legacy log detected (no per-frame timestamps).
   Upgrade to imu_logger2.2 for timestamped logs.

The data is still parsed correctly — you simply won’t have per-frame timestamps.

## 12. Sensor Mount & Axis Correction

The WT901 is mounted upside-down and flat in the vehicle (the “-Z up” configuration — the sensor’s Z axis points downward toward the ground). This is common when mounting in a tight enclosure. The sensor’s firmware does not fully compensate for this, so axis corrections must be applied in software.

### Physical orientation

Vehicle forward direction →

Top of enclosure (road-facing):  [normal bottom of PCB]
Bottom of enclosure (sky-facing): [normal top of PCB, connector side]

Sensor Z axis: points DOWN toward road (inverted from normal)
Sensor Y axis: longitudinal (forward/back), but INVERTED
Sensor X axis: lateral (left/right), OK

The raw roll reading in this mount is approximately -144° to -168°. This confirms the ~180° inversion.

### Correction formulas

# Applied in witmotion_log_parser-2.py and imu_logger4_API.py

accel_lat   =  raw_ax           # lateral: no change
accel_lon   = -raw_ay           # longitudinal: inverted
accel_vert  = -raw_az           # vertical: inverted

gyro_x      =  raw_gx
gyro_y      = -raw_gy           # inverted with Y
gyro_z      =  raw_gz

pitch       = -(raw_roll + 180) # correct for 180° flip
roll        =  raw_pitch
yaw         =  raw_yaw          # heading unaffected by Z flip

### Verification checklist

After installing in the car, verify corrections are correct by driving:

- Hard braking: accel_lon should go strongly positive (deceleration in forward direction).

- Hard acceleration: accel_lon should go negative.

- Left turn: roll should go positive (body rolls right = positive roll).

- Flat road, straight: pitch should be near 0°.

- Uphill: pitch should go positive.

If any of these are backwards, flip the sign on that axis correction.

## 13. Troubleshooting Log — Problems Solved

This section documents every significant issue encountered during development.

### Problem: witmotion-debug not found when run via systemd

Symptom: Service starts, but immediately fails with FileNotFoundError or command not found.

Root cause: systemd runs with a minimal PATH that does not include the venv’s bin/ directory. The witmotion-debug binary lives in the venv at /home/m1000/wt901/bin/witmotion-debug and is not on the system PATH.

Fix: The run_bootloader.sh wrapper exports the venv bin directory to PATH:

export PATH="/home/m1000/wt901/bin:$PATH"

And the service file also sets it:

Environment="PATH=/home/m1000/wt901/bin:/usr/bin:/bin"

Both layers are needed for full coverage.

### Problem: RPi.GPIO or witmotion not found despite being installed

Symptom: ModuleNotFoundError when running via systemd, but the script works fine when run manually.

Root cause: Running python3 script.py in the terminal uses the system Python or a different venv. The service must invoke the venv Python explicitly.

Fix: Always use the full absolute path to the venv Python in both the service file and any subprocess spawns:

PYTHON_BIN = "/home/m1000/wt901/bin/python"

Never use python3 or python in systemd-launched scripts — they will resolve to the wrong interpreter.

### Problem: Serial port silent — no data from IMU

Symptom: witmotion-debug runs but produces no output, or only state: idle -> idle.

Possible causes and fixes:

- Wrong port: Try both /dev/ttyAMA0 and /dev/ttyS0. Run ls /dev/serial* to see what’s available.

- Serial console not disabled: If the login shell is still attached to the UART, it will interfere. Re-run raspi-config and confirm the serial console is disabled.

- Bluetooth not disabled: If dtoverlay=disable-bt is not in config.txt, the hardware UART is still assigned to Bluetooth and the GPIO pins are not exposed as a usable serial port.

- User not in dialout group: Run groups to verify. Add with sudo usermod -aG dialout m1000 then log out and back in.

- Wrong baud rate: The WT901 defaults to 9600. Test with:

- stty -F /dev/ttyAMA0 9600 raw
cat /dev/ttyAMA0 | xxd | head

- You should see 0x55 bytes (the WT901 packet header).

### Problem: Bricking the WT901 (device stops responding)

Symptom: The IMU powered on but produced no serial output at any baud rate.

Root cause: The WT901’s baud rate was changed via a configuration command (either accidentally by a script or intentionally but then forgotten), and the host serial port was no longer configured to match.

Recovery procedure:

The WT901 can be recovered using the WitMotion PC software (Windows only) via USB-to-serial adapter, or by trying every possible baud rate in sequence:

for baud in 4800 9600 19200 38400 57600 115200 230400 460800; do
    echo "Trying $baud..."
    stty -F /dev/ttyAMA0 $baud raw
    timeout 2 cat /dev/ttyAMA0 | xxd | head -3
done

Once you find the baud rate that produces 0x55 bytes, you can send the command to reset it back to 9600 using the witmotion Python library:

from witmotion import IMU
imu = IMU(path='/dev/ttyAMA0', baudrate=<working_baud>)
imu.set_baudrate(9600)

Prevention: Do not change the WT901 baud rate unless you are certain you understand both the device-side and host-side configuration. If you do change it, update SERIAL_PORT and the --baud flag in the CMD list in imu_logger2.1.py.

### Problem: GPIO.cleanup() errors on logger restart

Symptom: When the logger exits and the bootloader tries to re-initialize GPIO, warnings about channels already in use or cleanup failing.

Root cause: If the logger crashes (as opposed to exiting cleanly via the STOP button), GPIO may not be properly cleaned up.

Fix: The bootloader’s finally block always calls GPIO.cleanup(). The re-initialization block after logger_process.wait() calls GPIO.setmode(GPIO.BCM) fresh. This pattern handles both clean exits and crashes.

### Problem: Python venv breaks after OS upgrade or apt upgrade

Symptom: ImportError or pip errors after a system update.

Root cause: System Python version changed; the venv was built against the old version.

Fix: Recreate the venv from scratch:

cd /home/m1000/wt901
rm -rf bin lib include pyvenv.cfg
python3 -m venv .
./bin/pip install witmotion RPi.GPIO

Your scripts and data files are unaffected — only the venv directories are deleted.

### Problem: select() read blocking the main loop

Symptom: Button presses are unresponsive during heavy data throughput.

Root cause: An earlier version of the logger used a blocking readline() call, which would hang if the subprocess paused or exited unexpectedly.

Fix: The current logger uses select.select() with a 0.1-second timeout:

ready, _, _ = select.select([process.stdout], [], [], 0.1)
if ready:
    line = process.stdout.readline()
else:
    continue

This guarantees the button check and LED blink run at least every 100ms.

### Problem: Parsed CSV has no timestamps (all blank)

Symptom: The timestamp_iso and time_hms columns in the parsed CSV are empty.

Root cause: The log was captured with an older version of the logger that did not prepend [HH:MM:SS.mmm] timestamp prefixes to each line.

Fix: Upgrade to imu_logger2.2+ which adds timestamps. Alternatively, accept that legacy logs will not have per-frame timestamps — the data values are still fully valid.

## 14. Calibration Procedure

The WT901 has onboard gyroscope and accelerometer calibration. This should be done when the sensor is first received, after any physical damage, or if angle drift is observed.

### Using imu_logger4_API.py (interactive menu)

/home/m1000/wt901/bin/python imu_logger4_API.py
# Enter port: /dev/ttyS0 (or /dev/ttyAMA0)
# Enter baud: 9600
# Choose option 4: Calibrate accel/gyro

The script will count down and then issue the calibration command. Keep the sensor completely still and level for the full 5 seconds.

### Manual calibration via witmotion library

from witmotion import IMU, CalibrationMode
imu = IMU(path='/dev/ttyAMA0', baudrate=9600)
imu.set_calibration_mode(CalibrationMode.AccelerometerGyroscopeCalibration)
import time; time.sleep(5)
imu.set_calibration_mode(CalibrationMode.None_)
imu.close()

### Notes on calibration

- Calibration results are stored in the WT901’s non-volatile memory. You do not need to recalibrate after power cycling.

- The magnetometer requires a separate calibration (figure-8 rotation in 3D space). This is less critical for vehicle dynamics data but matters for heading/yaw accuracy.

- Calibrate before mounting the sensor in the car. Once mounted, level calibration is difficult.

## 15. Testing the System

### Test 1: Manual run without systemd

Stop the service first, then run manually:

sudo systemctl stop wt901_bootloader.service
/home/m1000/wt901/bin/python /home/m1000/wt901/bootloadervw.py

BOOT_LED should turn on. ARMED_LED should blink. Press START — ARMED_LED should go solid briefly then off, and the logger should begin. Press STOP — logger should exit cleanly and the bootloader should return to blinking ARMED_LED.

### Test 2: Check raw log output

tail -f /home/m1000/wt901/witmotion_raw.log

While logging, you should see a continuous stream of acceleration, gyroscope, angle, and magnetic messages.

### Test 3: Run the parser

/home/m1000/wt901/bin/python /home/m1000/wt901/witmotion_log_parser-2.py

The terminal will print a summary table. Verify the frame count, Hz rate (~10 Hz at default settings), and that the corrected angle values make sense for the sensor’s current orientation.

### Test 4: Service auto-start

sudo systemctl enable wt901_bootloader.service
sudo reboot

After reboot, BOOT_LED should turn on within ~15 seconds of power-on. If it does not, check the journal:

sudo journalctl -u wt901_bootloader.service -b

### Test 5: Full drive test

Take the car out and log a short run with known maneuvers (hard stop, left turn, right turn, bump). Parse the log and verify that accel_lon, roll, and pitch all respond in the expected direction.

## 16. Future Improvements Roadmap

### GPS Integration

Adding GPS provides ground truth position and velocity data, enabling speed-correlated analysis of g-forces, corner entry speed, etc.

Recommended hardware: Any UART-based GPS module such as the u-blox M8N, PA1010D, or similar. The Pi has a second UART available (/dev/ttyAMA1) after the primary is assigned to the WT901, or you can use a USB GPS dongle.

Integration approach:

GPS UART → /dev/ttyAMA1 (or USB) → gpsd daemon → Python gpsd client

Install gpsd:

sudo apt install gpsd gpsd-clients python3-gps

In the logger, poll gpsd in a separate thread and merge GPS coordinates into the CSV at each IMU frame. Key fields to capture: latitude, longitude, altitude, speed (m/s), GPS timestamp (for time synchronization).

Synchronization note: IMU timestamps come from datetime.now() on the Pi. GPS provides UTC time. If the Pi has no internet (typical in a car), the system clock drifts. A GPS-locked time sync via gpsd + chrony will keep timestamps accurate even without WiFi.

### Live Telemetry Radio (LoRa or Cellular)

For real-time data transmission to a pit laptop, two approaches are practical at the SAE Baja scale:

Option A: LoRa radio (recommended for off-road use)

LoRa (Long Range) radio modules operate at 915 MHz in the US and achieve 1–10 km range even through terrain and buildings — ideal for Baja courses.

- Hardware: RFM95W modules, or purpose-built LoRa HATs (e.g., the WaveShare SX1268 HAT).

- Library: lora-from-scratch or the pyLoRa / sx126x Python libraries.

- Protocol: Send compressed JSON or binary-packed structs of the last 5 frames every 200ms. Do not send every frame — the LoRa data rate is low (~5 kbps at maximum range).

Option B: Cellular (LTE)

If your race area has cell coverage, a USB LTE dongle (e.g., Huawei E3372, or a Sixfab 4G HAT) provides higher bandwidth for near-real-time streaming.

- Stream to a cloud endpoint (simple Flask or FastAPI server).

- The Pi sends POSTed JSON payloads every 500ms.

General telemetry architecture:

imu_logger (thread) → telemetry_queue → radio_sender_thread → LoRa/LTE

Keep the telemetry sending in a separate daemon thread with a bounded queue. If the radio is busy or out of range, frames are dropped from the queue rather than backing up and crashing the logger.

### Web Dashboard / Visualization

A live dashboard visible to pit crew on a laptop or tablet.

Approach: Run a lightweight web server on the Pi itself. The Pi serves a page over WiFi (either connected to a hotspot or running its own access point with hostapd).

Recommended stack:

- Backend: Python FastAPI or Flask with a WebSocket endpoint.

- Frontend: Plain HTML + JavaScript using Chart.js or similar for live graphs.

- The logger pushes new frames to a shared in-memory buffer; the WebSocket handler streams them to connected clients.

Minimal example architecture:

# In the logger, after writing each CSV row:
telemetry_buffer.append({
    "t": ts_ms,
    "ax": c_accel[0], "ay": c_accel[1], "az": c_accel[2],
    "roll": c_angle[1], "pitch": c_angle[0], "yaw": c_angle[2],
})

# In a separate FastAPI app (run in another thread or process):
@app.websocket("/ws")
async def websocket_endpoint(ws):
    while True:
        if telemetry_buffer:
            await ws.send_json(telemetry_buffer[-1])
        await asyncio.sleep(0.1)

Pi as WiFi access point: Useful when the car is out of WiFi range. Install hostapd and dnsmasq, configure the Pi to create a network called (e.g.) BAJA-TELEMETRY. Pit crew connect directly and browse to http://192.168.4.1/.

### Additional Sensor Channels

With the serial and GPIO infrastructure in place, additional sensors are straightforward to add:

- Suspension potentiometers: Analog via ADS1115 (I2C ADC), 4 channels for 4-corner suspension travel.

- Wheel speed sensors (hall effect): GPIO interrupt-based pulse counting. RPM from pulse timing, speed from wheel circumference.

- Engine RPM: Tap into ignition signal, same approach as wheel speed via GPIO interrupt.

- CAN bus (future): If the vehicle gains a CAN-capable ECU, the Pi can read it via MCP2515 SPI CAN controller.

### Post-Processing Improvements

- Automatic session splitting: The parser currently handles multi-session logs. Auto-split on SESSION_START markers for per-run CSV files.

- G-force event detection: Flag frames where accel_lon or roll exceeds a threshold — automatically highlight hard corners, jumps, and impacts.

- Lap timing: If a GPS waypoint is set for the start/finish, detect lap crossings and split the CSV into per-lap segments.

## 17. Quick Reference Cheat Sheet

### Starting and stopping the service

sudo systemctl start wt901_bootloader.service
sudo systemctl stop wt901_bootloader.service
sudo systemctl restart wt901_bootloader.service
sudo systemctl status wt901_bootloader.service
sudo journalctl -u wt901_bootloader.service -f

### Manual run (debug mode)

sudo systemctl stop wt901_bootloader.service
/home/m1000/wt901/bin/python /home/m1000/wt901/bootloadervw.py

### Test IMU serial directly

/home/m1000/wt901/bin/witmotion-debug --path /dev/ttyAMA0 --verbose

### Parse a log file

/home/m1000/wt901/bin/python /home/m1000/wt901/witmotion_log_parser-2.py
# or with explicit paths:
/home/m1000/wt901/bin/python /home/m1000/wt901/witmotion_log_parser-2.py witmotion_raw.log run1_parsed.csv

### Reinstall venv from scratch

cd /home/m1000/wt901
rm -rf bin lib include pyvenv.cfg
python3 -m venv .
./bin/pip install witmotion RPi.GPIO

### Check serial ports

ls /dev/ttyAMA* /dev/ttyS* /dev/serial*
# Test data at 9600 baud:
stty -F /dev/ttyAMA0 9600 raw && timeout 3 cat /dev/ttyAMA0 | xxd | head -10

### GPIO pin reference (quick)

GPIO 13  → START button (other leg → GND)
GPIO 17  → STOP button  (other leg → GND)
GPIO 16  → SHUTDOWN button (other leg → GND)

GPIO 19  → ARMED LED    — 1 Hz blink = waiting for start
GPIO 21  → LOG LED      — 4 Hz blink = actively logging
GPIO 26  → BOOT LED     — solid = bootloader alive
GPIO 18  → SHUTDOWN LED — 10 Hz blink = shutdown in progress

GPIO 14 (TXD) → Yellow wire → WT901 RX
GPIO 15 (RXD) → Green wire  ← WT901 TX
GND terminal  → Purple wire → WT901 GND
5V terminal   → Red wire    → WT901 VCC

Document prepared March 2026. System built and validated on Raspberry Pi 4B (8 GB), Raspberry Pi OS Lite 64-bit, WitMotion WT901 firmware version 18159.

SAMPLE OUTPUTS: AFTER OFFLOAD -> ANALYSIS: